| EXP:3 | Setting Up Hybrid Cloud Connectivity Using Cisco Packet Tracer |
| --- | --- |
| 22/7/2026 |  |

# AIM

To simulate a secure hybrid cloud connection between an on-premises data center and a cloud network using routers, switches, subnets, and routing in Cisco Packet Tracer.

# SOFTWARE / COMPONENTS REQUIRED

- Cisco Packet Tracer

- Two Cisco 2911 routers

- Two Cisco 2960-24TT switches

- One PC

- One Server

- Copper Straight-Through cables

- Copper Cross-Over cable for the router-to-router link

# NETWORK TOPOLOGY

The simulated network consists of an on-premises LAN connected to a cloud/server LAN through two routers. Static routing is used between the two networks.

| PC0 | → | Switch0 | → | Router1 |
| --- | --- | --- | --- | --- |
| Router1 | → | 10.10.10.0/30 | → | Router2 |
| Router2 | → | Switch1 | → | Server0 |
| PC Network | → | 192.168.1.0/24 |  |  |
| Server Network | → | 10.0.0.0/24 |  |  |

# IP ADDRESSING TABLE

| Device | Interface | IP Address | Subnet Mask | Purpose |
| --- | --- | --- | --- | --- |
| Router1 | GigabitEthernet0/0 | 192.168.1.1 | 255.255.255.0 | On-premises LAN gateway |
| Router1 | GigabitEthernet0/1 | 10.10.10.1 | 255.255.255.252 | Router-to-router link |
| Router2 | GigabitEthernet0/0 | 10.0.0.1 | 255.255.255.0 | Cloud LAN gateway |
| Router2 | GigabitEthernet0/1 | 10.10.10.2 | 255.255.255.252 | Router-to-router link |
| PC0 | FastEthernet0 | 192.168.1.10 | 255.255.255.0 | On-premises host |
| Server0 | FastEthernet0 | 10.0.0.10 | 255.255.255.0 | Cloud server |

# PROCEDURE

- Open Cisco Packet Tracer and create a new logical workspace.

- Place two 2911 routers, two 2960 switches, one PC and one Server.

- Connect PC0 to Switch0 and Switch0 to Router1 using Copper Straight-Through cables.

- Connect Server0 to Switch1 and Switch1 to Router2 using Copper Straight-Through cables.

- Connect Router1 GigabitEthernet0/1 to Router2 GigabitEthernet0/1 using a Copper Cross-Over cable.

- Open Router1 CLI. When prompted for the initial configuration dialog, enter no and press Enter.

- Configure Router1's LAN and inter-router interfaces.

- Verify Router1 interfaces using show ip interface brief.

- Open Router2 CLI and configure its LAN and inter-router interfaces.

- Configure PC0 with IP address 192.168.1.10, subnet mask 255.255.255.0 and default gateway 192.168.1.1.

- Configure Server0 with IP address 10.0.0.10, subnet mask 255.255.255.0 and default gateway 10.0.0.1.

- Configure static routes on both routers so that the two LANs can communicate.

- Verify the routing tables using show ip route.

- From PC0, use ping 10.0.0.10 to verify end-to-end connectivity.

- Save the router configurations using copy running-config startup-config.

# ROUTER CONFIGURATION COMMANDS

## Router1 Configuration

enable

configure terminal

interface gigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown exit

interface gigabitEthernet0/1

ip address 10.10.10.1 255.255.255.252

no shutdown exit

ip route 10.0.0.0 255.255.255.0 10.10.10.2

exit

copy running-config startup-config

## Router2 Configuration

enable

configure terminal

interface gigabitEthernet0/0

ip address 10.0.0.1 255.255.255.0 no shutdown

exit

interface gigabitEthernet0/1

ip address 10.10.10.2 255.255.255.252

no shutdown exit

ip route 192.168.1.0 255.255.255.0 10.10.10.1

exit

copy running-config startup-config

## Verification Commands

do show ip interface brief show ip route

ping 10.0.0.10

# OUTPUT / OBSERVATION

The following outputs were obtained during the simulation.

## Router1 Interface Verification

| Interface | IP-Address | Status | Protocol |  |  |
| --- | --- | --- | --- | --- | --- |
| GigabitEthernet0/0 | 192.168.1.1 | up | up |  |  |
| GigabitEthernet0/1 | 10.10.10.1 | up | up |  |  |
| GigabitEthernet0/2 | unassigned | administratively |  | down | down |
| Vlan1 | unassigned | administratively |  | down | down |

## Router2 Interface Verification

| Interface | IP-Address | Status | Protocol |  |  |
| --- | --- | --- | --- | --- | --- |
| GigabitEthernet0/0 | 10.0.0.1 | up | up |  |  |
| GigabitEthernet0/1 | 10.10.10.2 | up | up |  |  |
| GigabitEthernet0/2 | unassigned | administratively |  | down | down |
| Vlan1 | unassigned | administratively |  | down | down |

## Router1 Routing Table – Static Route

S 10.0.0.0/24 [1/0] via 10.10.10.2

C 10.10.10.0/30 is directly connected, GigabitEthernet0/1 L 10.10.10.1/32 is directly connected, GigabitEthernet0/1 C 192.168.1.0/24 is directly connected, GigabitEthernet0/0 L 192.168.1.1/32 is directly connected, GigabitEthernet0/0

## Router2 Routing Table – Static Route

C 10.0.0.0/24 is directly connected, GigabitEthernet0/0 L 10.0.0.1/32 is directly connected, GigabitEthernet0/0

C 10.10.10.0/30 is directly connected, GigabitEthernet0/1 L 10.10.10.2/32 is directly connected, GigabitEthernet0/1 S 192.168.1.0/24 [1/0] via 10.10.10.1

## Connectivity Test

C:\>ping 10.0.0.10

Pinging 10.0.0.10 with 32 bytes of data:

Reply from 10.0.0.10: bytes=32 time<1ms TTL=126 Reply from 10.0.0.10: bytes=32 time<1ms TTL=126 Reply from 10.0.0.10: bytes=32 time<1ms TTL=126 Reply from 10.0.0.10: bytes=32 time<1ms TTL=126

Ping statistics for 10.0.0.10:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss) Minimum = 0ms, Maximum = 1ms, Average = 0ms

# CONCLUSION

The experiment demonstrated the configuration of a basic hybrid cloud connectivity scenario using routers, switches, an end device and a server. Static routes enabled traffic to travel between the 192.168.1.0/24 and 10.0.0.0/24 networks.